Using BananaCom

When BananaCom is started, you immediately see "the main terminal

screen". The bottom line gives you some information about current

BananaCom settings and will try to direct you to keys that will help

you do things.  The rest of the screen is the space that your

favorite modem service will use to dazzle you!

As with many programs, your escape key helps you to "back up" or

"get out" of things.  The only time that escape will not give you

this effect is at the main terminal screen.  The reason is that the

escape key is sent to the modem service!  Instead, you must use

Alt-X (to exit BananaCom).

Dialing Directory (Alt-D)

From the dialing directory, use your arrow keys to select a modem

service and press enter.  BananaCom will then dial the selected

service. In a few seconds your modem will connect to the service.

Note that pressing extra keys does not speed things up, it hangs up

the phone!

If you want to add a modem service to the dialing directory, go to

the bottom of the directory and press enter.  Fill in the fields for

the name and phone number (be sure that you don't use a voice or fax

number!).  Press your F2 key to save the changes.

ZModem file transfers

Receiving (downloading):  Tell the modem service which file(s) you

want to receive via ZModem.  BananaCom will detect the transfer

request and put a box on your screen to show you the progress of the

file transfer.  When an exact copy of the file is in your computer,

the box will be taken away and you will be able to continue with

using your modem service. When your modem service session is over,

you can exit from BananaCom (with Alt-X) and work with your

downloaded file just as you would with any other file!

Sending (uploading):  When you tell your modem service that you want

to upload via ZModem, BananaCom will detect the transfer request and

ask you for a file name.  Type the file name, press enter and the

upload will begin.  If your modem service is an odd duck, it might

wait dumbly for you to activate the transfer. If this happens, press

Alt-M and select "ZModem Upload".

Hang Up (Alt-H)

When you are done using a modem service, follow the instructions of

the service for terminating the call.  If you seem to be stuck or

cannot find a way to have the service terminate the call, press

Alt-H to hang up the phone on your end.

Toggle Text Capture (Alt-T)

Tap Alt-T once.  Now all of the new text that appears on the screen

will be appended to the file CAPTURE.TXT.  Tap Alt-T again and text

capture will stop.

This is especially useful if you want to print something from a

modem service.  Capture what you want to print and open CAPTURE.TXT

in your favorite word processor.  Now you can edit and print the

captured text just as you would any other document!

Upload Text (Alt-U)

When you have prepared the modem service to receive a message, type

Alt-U and provide the name of your text file.  BananaCom will pass

the message very quickly.  The modem service will have no idea that

there is a text upload in progress - it will think that you are a

very fast typist!

Back scroll terminal screen (Alt-B)

Sometimes when using a modem service, you find that information you

need has scrolled past the top of the screen!  To look at that

information again, press Alt-B.  Use your cursor keys to view the

text that has recently passed.

Troubleshooting

The most common problem is improperly configured hardware. Press

Alt-Q to verify hardware quality.

Some modems are picky about which plug is for the phone line that

goes to the wall and which one goes to the telephone.  If you

suspect this could be the problem, swap the plugs!

External Modems

Check to make sure the serial cable is tightly connected to the

modem and your computer.  You MUST use a cable that has at least 9

wires in it (pins 1 through 8 and pin 20). Note that if you are

using a 9 pin to 25 pin converter, some converters out there do

the conversion wrong!

If you are trying to go faster than 9600bps and your modem's

serial port does not have a 16550A chip, you could have problems.

Set your speed to 9600 and see if things improve.

Pentium 2

Intel has confessed that there is a problem with some of their

P2 chips and DOS com software.  If BananaCom locks up when

activated, it is either a com port conflict or else the dreaded

P2 problem.  Contact Intel.

Laptops

Many laptops save power by shutting the modem off when not in use.

See your laptop manual to find out how you turn your modem on.

Some PCMCIA modems require a special DOS driver for the modem to

work.  See your modem manual for details.

Windows 95

Some fax, modem or voice programs in Windows 95 will hog the fax

modem so that nothing else can use it.  You will have to shut down

those programs before BananaCom will have access to your hardware.

Plug-N-Play modems

Make sure that you used the special activation software that came

with the modem to set it up for DOS programs.

WinModems

DOS software does not work with WinModems.

Garbage (unwanted characters) on the screen could be caused by:

a) a low quality or RPI modem.

b) somebody picking up the phone while you are using the modem.

c) line noise from the phone company or long distance carrier.

d) too much "flat wire" - the wire that runs from the phone jack

to your phone is usually flat and acts as a sort of antennae -

the more you have, the more noise you get.  Flat wire close to

lights or electric motors makes things worse.

e) there is another phone/device connected to this line

contributing noise.  Unplug all unneeded devices at the wall.

f) call waiting solutions:

1) Call your phone company and permanently cancel call waiting.

2) Temporarily cancel call waiting by changing your dialing

prefix to "ATDT*70," (most phone companies support this).

g) Some phone companies have been cutting the quality of their

phone line service - voice calls will still sound okay, but

modems and fax machines cannot tolerate this.

Technical information (for experts only):

BananaCom uses:  ANSI/VT100 terminal emulation; no echo, full

duplex, no CR/LF conversion (except in text uploads); destructive

backspace and direct screen writes.

BananaCom will use a 16550A FIFO buffer, if found.

Flow control

If BananaCom cannot establish RTS/CTS flow control, it will issue

a warning and continue on with no flow control.

XON/XOFF can only be activated through the dialing directory.

E71

E71 (Even parity, 7 data bits, 1 stop bit) can only be activated

through the dialing directory.

Dialing Directory Macros

If you use calling cards without using macros, your dialing

directory might look like this:

Montana Banana    0-406-543-8234,,,,40654319289999

If you created BCOMMACS.TXT with the line

CC ,,,,40654319289999

your dialing directory could now look like this

Montana Banana                  0-406-543-8234(CC)

Keyboard Macros

Create a text file BCOM.MAC with up to ten lines of text.  The

first line is what will be sent when you press Alt-F1.  The second

line matches up to Alt-F2.  Etc.  ^M will send a return and ^[

will send an escape.

Download Directory

If you want to direct all of your downloads to a particular

directory, create the file DOWNLOAD.TXT with that directory name

in the first line.

Note that you might be more satisfied with the results of "Running

from any directory" (see below for details).

Upload Directory

Create UPLOAD.TXT with a directory name in the first line.  This

will now be the default during uploads.

Running from any directory

Using the PATH and BCOM environment variables, you can put your

BananaCom files in one directory and run BananaCom from another.

To do this, make these changes to your AUTOEXEC.BAT file:

1)  Make sure the BananaCom directory is in your PATH statement

2)  Include   SET BCOM=<BananaCom directory>

Example:  SET BCOM=C:\MODEM

Make sure there is no BCOM.BAT somewhere else in your path!

Doorway mode can be toggled with the Alt-= key.

Files of interest at www.split.com

BCUSTOM.ZIP   BananaCom Custom - auto dials one BBS

BABYB.ZIP     Baby Banana - BananaCom Custom shareware - $49!

BSYSOP.ZIP    Tips and info on distributing BananaCom

SCRIPT.DOC    Documentation for the script language

BCOML.ZIP     BananaCom Lean uses less memory. Good for palmtops.

BCOM*.ZIP     BananaCom in French, Spanish, and other languages.

To get support directly from the author, send your voice phone

number or e-mail address along with a check for $30 to

Paul Wheaton

17937 East Amherst Avenue

Aurora, CO  80013

Legaleeze

Copyright (C) 1994, 1998 by Paul Wheaton, Denver, Colorado, USA.

All rights reserved.

Licensed under U.S. Patent No. 4,558,302 and foreign counterparts.

All distribution of BananaCom is heartily encouraged as long as the

EXE file is not modified.

THIS PROGRAM IS PROVIDED "AS-IS" AND WITHOUT WARRANTY OF ANY KIND.

BANANA PROGRAMMING DISCLAIMS ANY AND ALL OTHER WARRANTIES, EXPRESS

OR IMPLIED, ORAL OR WRITTEN, INCLUDING ANY IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

IN NO EVENT SHALL BANANA PROGRAMMING BE LIABLE FOR ANY INCIDENTAL,

CONSEQUENTIAL, OR PUNITIVE DAMAGES WHATSOEVER ARISING OUT OF USE OF

THE PROGRAM OR YOUR RELATIONSHIP WITH BANANA PROGRAMMING, INCLUDING

WITHOUT LIMITATION ANY OR ALL DAMAGES FOR LOSS OF PROFITS, BUSINESS

INTERRUPTION, LOSS OF INFORMATION OR ANY PECUNIARY LOSS, EVEN IF

BANANA PROGRAMMING HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES.